©2009 RealTEK Semiconductor Corp. All rights reserved. No part of this document may be reproduced, transmitted, transcribed, stored in a retrieval system, or translated into any language in any form or by any means without the written permission of RealTEK Semiconductor Corp.

RTL8188E Mass Production DLL Specification

# Document Number:  01-001-M4-0004

Naming Rule: Reference Internal Engineer Development Document Code

Example: 01 0001 H1 01

Serial Number

RTL819X

Wireless NIC

High Level Design Firmware Architecture

Document Code Management

|  |  |  | Code |
| --- | --- | --- | --- |
| RTL Document |  | Functional Spec | F1 |
|  |  | High Level Design | H1 |
|  |  | Low Level Design | L1 |
|  |  | Test Plan, Test Report | T1 |
|  |  | Guideline | G1 |
|  |  | Maintain | M1 |
|  |  | Decisions , Meeting Notes | D1 |

Realtek Semiconductor Corp.

Headquarters

1F, No. 2, Industry East Road IX, Science-based

Industrial Park, Hsinchu, 300, Taiwan, R.O.C.

Tel: 886-3-5780211  Fax: 886-3-5776047

http://www.realtek.com.tw/

# Change History

| Version | Editor | Date | Remarks |
| --- | --- | --- | --- |
| 0.001 | MH | 2009/01/09 | - Merge from previoud DLL_Function_Specification.doc - Add new API for 92S series. - Add efuse operation API in chapter 4. |
| 0.002 | Cosa | 2009/02/26 | - Add dll function for 92S |
| 0.003 | chiyoko | 2011/08/19 | - For 8723A |
| 0.004 | chiyoko | 2011/10/21 | - Revise BT Efuse API |
| 0.005 | chiyoko | 2011/11/10 | - For 8188E |

# Table of Contents:

Document Number:  01-001-M4-0001	2

Change History	3

Table of Contents:	4

List of Figure	5

List of Table	6

1.	MP DLL Introduction	7

2.	General Functions	8

3.	EEPROM Read/Write Functions	12

4.	EFUSE Operation Functions	14

5.	Packet TX/RX Functions	15

6.	Conclusion	17

# List of Figure

Figure 1.	Porting Sequence	7

# List of Table

Table 1.	DLL API Name Table	7

# MP DLL Introduction

## DLL Table

- DLL API Name Table

| Direction | API Name | Remark |
| --- | --- | --- |
| General |  |  |
|  | StartPacketDriver |  |
|  | StopPacketDriver |  |
|  | OpenDUT |  |
|  | CloseDUT |  |
|  | StartTest |  |
|  | StopTest |  |
|  | SetDataRate |  |
|  | SetChannelDirectCall |  |
|  | SetTxContinuousDirectCall |  |
|  | SetCarrierSuppressionTxContinuous |  |
|  | SetPreamble |  |
|  | SetTxPowerControl |  |
|  | ReadMACAddress |  |
|  | WriteMACAddress |  |
|  | QueryChipIDandVer |  |
|  | QuerySignalLocationType |  |
|  | SetSignalLocationType |  |
|  | ReadRFThermalMeter |  |
|  | DLL_SwitchTxPwrTrack |  |
|  | DLL_QueryTxPwrTrack |  |
|  | DLL_CheckAutloadStatus |  |
|  | DLL_ReadEFuseUtilization |  |
|  | LEDControl |  |
|  | ButtonPushed |  |
|  | QueryRfStatus |  |
|  | GetCalTxPwrIndex |  |
|  | SetRfPathSwitch |  |
|  | GetRfPathSwitch |  |
|  | SetRFOnOff |  |
|  | SetCrystalCap |  |
| EEPROM |  |  |
|  | QueryEEpromType |  |
|  | ReadEEprom |  |
|  | ReadEEpromByte |  |
|  | WriteEEprom |  |
|  | WriteEEpromByte |  |
| EFUSE |  |  |
|  | ReadEFuse |  |
|  | ReadEFuseByte |  |
|  | WriteEFuse |  |
|  | WriteEFuseByte |  |
|  | UpdateEFuse |  |
| Packet TX/RX |  |  |
|  | ResetTxPacketSent |  |
|  | QueryTxPacketSent |  |
|  | ResetRxPacketReceived |  |
|  | QueryRxPacketReceived |  |
|  | QueryRxPacketCRC32Error |  |
|  | PacketAllocatePacket |  |
|  | PacketInitPacket |  |
|  | PacketSendPacket |  |
|  | PacketWaitPacket |  |
|  | PacketFreePacket |  |

# General Functions

- BOOL StartPacketDriver (LPTSTR ServiceName, BOOL bReloadService=FALSE)
This function attempts to start the service of the protocol driver specified by ServiceName. If bReloadService is set to true and the service has already started, it will stop and then start the service.

- <Note>

- 1. It MUST be called first before using other API exported in this DLL.

- 2. ServiceName must be an UNICODE string.

- <Example>

TCHAR szServiceName[64] = _T("mp8185s");

wchar_t wszServiceName[64];

ifndef _UNICODE

swprintf(wszServiceName, L"%S", szServiceName);// Translate char to wchar

#else

swprintf(wszServiceName, L"%s", szServiceName);

#endif

StartPacketDriver((char*)wszServiceName, TRUE);

- BOOL StopPacketDriver (LPTSTR ServiceName)
This function attempts to stop the service of the protocol driver, note that, ServiceName must be an UNICODE string.

- <Note>

- 1. It should be invoked before your program terminating.

- 2. ServiceName must be an UNICODE string.

- <Example>

TCHAR szServiceName[64] = _T("mp8185s");

wchar_t wszServiceName[64];

ifndef _UNICODE

swprintf(wszServiceName, L"%S", szServiceName);// Translate char to wchar

#else

swprintf(wszServiceName, L"%s", szServiceName);

#endif

StopPacketDriver((char*)wszServiceName, TRUE);

- PVOID OpenDUT(LPTSTR AdapterName)
This function opens the connection with DUT and get the handle of the DUT. It will return a NULL pointer if specifed AdapterName is invalid, otherwise, it will allocate memory for a ADAPTER object and return a pointer to this object. Other function who needs the argumenent, LPADAPTER AdapterObject, shall use the pointer returned by OpenDUT().

- <Note>

- 1. It should be invoked before other API needs the input argument, LPADAPTER AdapterObject.

- 2. AdapterName must be an UNICODE string.

- 3. The format of AdapterName is \Device\mp8185s_GUID.

- int CloseDUT(LPADAPTER AdapterObject)
This function closes the connection with DUT and release the handle and memory block allocated in OpenDUT() for the DUT. It will return TRUE/FALSE to show operation success or fail.

- <Note>

- 1. It MUST be invoked when we don’t need to access the AdapterObject any more before terminating the calling program. Otherwise, a memory leak problem will be encounted.

- <Example>

//

- // wchar wszAdapterName = L”\Device\mp8185s_{08E0D84D-1B31-4D32-93B2-0AE05C90192D}”;

- //

- LPADAPTER pAdapter =OpenDUT(wszAdapterName);

- StartTest(pAdapter);

- StopTest(pAdapter);

- CloseDUT(pAdapter);

- int StartTest(LPADAPTER AdaterObject)
This function initializes the DUT to the MP test mode. It will return TRUE/FALSE to show operation success or fail.

- <Note>

- 1.We SHOULD invoked this function before other Mass Production related test item, for example, Packet Tx test, Packet Rx test, Continuous Tx test, Single Carrier Tx test, Carrier Suppression Tx test and so on .

- int StopTest(LPADAPTER AdapterObject)
This function change the DUT from the test mode into normal mode. It will return TRUE/FALSE to show operation success or fail.

- . <Note>

- 1.This function MUST be invoked if StartTest() is had been called before.

- int SetDataRate(LPADAPTER AdapterObject, int Rate)
This function sets the transmit data rate of the DUT. Available data rates are: 
 1       : CCK 1M

- 2       : CCK 2M

- 3       : CCK 5.5M

- 4       : CCK 11M

- 5       : OFDM 6M

- 6       : OFDM 9M

- 7       : OFDM 12M

- 8       : OFDM 18M

- 9       : OFDM 24M

- 10      : OFDM 36M

- 11      : OFDM 48M

- 12      : OFDM 54M

- 13~28   : MCS0~MCS15It will return TRUE/FALSE  to show operation success or fail.

- int SetChannelDirectCall(LPADAPTER AdapterObject, int Channel)
This function sets the DUT to transmit and receive on a specific channel. It will return TRUE/FALSE to show operation success or fail..

- int SetTxContinuousDirectCall(LPADAPTER AdapterObject, int Mode)
This functions starts(Mode: 1) or stops(Mode: 0) the DUT to perform continuous transmission. It will return TRUE/FALSE to show operation success or fail.

- int SetCarrierSuppressionTxContinuous(LPADAPTER AdapterObject, int Mode)
This functions starts(Mode: 1) or stops(Mode: 0) the DUT to perform carrier suppression continuous transmission. It will return TRUE/FALSE to show operation success or fail.

- int SetPreamble(LPADAPTER AdapterObject, int Mode)
This function sets the DUT to use long preamble(Mode: 1) or short preamble(Mode: 2), or Long GI(Mode: 3), or Short GI(Mode: 4). It will return TRUE/FALSE to show operation success or fail.

- Int SetTxPowerControl(LPADAPTER AdapterObject, int Value)
This function sets the DUT’s TX POWER CONTROL of the current channel to the desired value, that is, it changes current setting of hardware, and corresponding field in EEPROM is not changed in this function. It will return TRUE/FALSE to show operation success or fail.

- int ReadMACAddress(LPADAPTER AdapterObject, char* MACAddress)
This function reads the MAC address from the DUT MAC registers. It will return TRUE/FALSE to show operation success or fail.

- int WriteMACAddress(LPADAPTER AdapterObject, char* MACAddress)
This function reads the MAC address from the DUT MAC registers. It will return TRUE/FALSE to show operation success or fail.

- int QueryChipIDandVer(LPADAPTER AdapterObject)
This function queries the Chip ID and Version from the driver. It will return TRUE/FALSE to show operation success or fail.

- int QuerySignalLocationType (LPADAPTER AdapterObject, int * SignalLocation)
This function queries the Signal Location, 20M(Mode 0), 40MHZ_DUPLICATED(Mode 1), 40MHZ_LOWER(Mode 2), 40MHZ_UPPER(Mode 3), 40MHZ_DSC(Mode 4). It will return TRUE/FALSE to show operation success or fail.

- int SetSignalLocationType (LPADAPTER AdapterObject, int SignalLocation)
This function sets the Signal Location, 20M(Mode 0), 40MHZ_DUPLICATED(Mode 1), 40MHZ_LOWER(Mode 2), 40MHZ_UPPER(Mode 3), 40MHZ_FULL(Mode 4). It will return TRUE/FALSE to show operation success or fail. (NOTE: only 20M(Mode 0) and 40M(Mode 4) are supported now.

- int ReadRFThermalMeter (LPADAPTER AdapterObject, unsigned int *ThermalValue)
This routine reads back the Thermal Meter value. Bit0~15 is the value of RF-A and bit16~31 is the value of RF-C.

- int DLL_SwitchTxPwrTrack(LPADAPTER AdapterObject, unsigned char TxPwrTrackState)

This routine switch Tx Power Track state. It will return TRUE/FALSE to show operation success or fail.

- int DLL_QueryTxPwrTrack (LPADAPTER AdapterObject, int *TxPwrTrackState)

This routine query Tx Power Track state. It will return TRUE/FALSE to show operation success or fail.

- int DLL_CheckAutloadStatus(LPADAPTER AdapterObject, unsigned char *AutoloadFail)

This routine check autoload status fail or success. 1=Autoload Fail, 0=Autoload Success.It will return TRUE/FALSE to show operation success or fail.

- int DLL_ReadEFuseUtilization(LPADAPTER AdapterObject, unsigned int *EFuseUtilize)

This routine get the EFUSE utilization, bit0~15 for how many bytes used, bit 16~31 for percentage. It will return TRUE/FALSE to show operation success or fail.

- BOOLEAN LEDControl(LPADAPTER AdapterObject, int led_num, int led_mode)

This routine sets the led_num to light as the led_mode. Led_num=0~1, LED OFF:set led_mode=0, LED ON: set led_mode=1. It will return TRUE/FALSE to show operation success or fail.

- BOOLEAN ButtonPushed(LPADAPTER AdapterObject, int buttonType, int *pushed)

This routine returns the button pushed or not. WPS Button: buttonType=0. pushed=1 if the button is pushed, pushed=0 means the button is not pushed. It will return TRUE/FALSE to show operation success or fail.

- BOOLEAN QueryRfStatus(LPADAPTER AdapterObject, int *rfstatus)

This routine returns the HW RF on/off status. rfstatus=2 if HW RF off, pushed=0 means HW RF on. It will return TRUE/FALSE to show operation success or fail.

- BOOLEAN GetCalTxPwrIndex(LPADAPTER AdapterObject, int *powerIndex)

- This routine returns the real calculated Tx power index by current channel and rate in driver config. It will return TRUE/FALSE to show operation success or fail.

- INT SetRfPAthSwitch(LPADAPTER AdapterObject, BOOL bMain)

- This routine sets the RF path to Main or Aux. bMain = TRUE means set RF path to Main, bMain = FALSE means set RF path to Aux. It will return TRUE/FALSE to show operation success or fail.

- INT GetRfPAthSwitch(LPADAPTER AdapterObject, BOOL *bMain)

- This routine gets the RF path setting. bMain = TRUE means RF path is set to Main, bMain = FALSE means RF path is set to Aux. It will return TRUE/FALSE to show operation success or fail.

- BOOLEAN SetRFOnOff(LPADAPTER AdapterObject, int RfOnOff )

- This routine set RF on/off (//0: on, 1: off). Set Rf off before start BT test and set RF on after BT test finished.

- BOOLEAN SetCrystalCap(LPADAPTER AdapterObject, int CrystalCap  )

- This routine set Crystal Cap value (0x00~0x3F). It will return TRUE/FALSE to show operation success or fail.

# EEPROM Read/Write Functions

- int QueryEEpromType(LPADAPTER AdapterObject, int* Type)
This function queries the type of EEPROM in the DUT. It will return TRUE/FALSE to show operation success or fail.

- <Note>

- 1.Valid range of the argument Type:

1: 128-byte EEPROM

2: 256-byte EEPROM

3: EFUSE

- 2. ReadEEprom() and WriteEEPROM() should input proper Type from the result of QueryEEpromType().

- int WriteEEprom(LPADAPTER AdapterObject, char* EEprom, int Type)
This function writes the EEPROM content of length 128 or 256 bytes into the EEPROM of DUT. It will return TRUE/FALSE to show operation success or fail.

- <Note>

- 1.Valid range of the argument Type:

1: 128-byte EEPROM

2: 256-byte EEPROM

- <Example>

- // LPADAPTER pAdapter;

- // int Type;

- // UCHAR EEprom[256];

- if(QueryEEpromType(pAdapter, &Type))

- {
   WriteEEprom(pAdapter, EEprom, Type);

- }

- int WriteEEpromByte(LPADAPTER AdapterObject, UCHAR offset, UCHAR Value)
This function writes 1-byte value into the byte offset of EEPROM. It will return TRUE/FALSE to show operation success or fail.

- int ReadEEprom(LPADAPTER AdapterObject, char* EEprom, int Type)
This function reads the EEPROM content of length 128(93c46, Type: 0) or length 256(93c56, Type:1 ) from the DUT EEPROM. It will return TRUE/FALSE to show operation success or fail.

- <Note>

- 1.Valid range of the argument Type:

1: 128-byte EEPROM

2: 256-byte EEPROM

- <Example>

- // LPADAPTER pAdapter;

- // int Type;

- // UCHAR EEprom[256];

- int ReadEEpromByte(LPADAPTER AdapterObject, UCHAR offset, UCHAR* Value)
This function reads the value of the byte offset specified from EEPROM. It will return TRUE/FALSE to show operation success or fail

# EFUSE Operation Functions

- int RTLMP_API  ReadEFuse(LPADAPTER AdapterObject, char* EFuse)
This function support to read EFUSE shadow memory from NIC side. You can not read real Efuse content. Driver will assist to protect you to execute incorrect operation on EFUSE area.

- int RTLMP_API	ReadEFuseByte(   LPADAPTER   AdapterObject,

- UCHAR       Offset,

- UCHAR*      Value    )
This function reads the value of the byte offset specified from EFUSE shadow memory. It will return TRUE/FALSE to show operation success or fail

- int RTLMP_API	WriteEFuse(    	LPADAPTER   AdapterObject,

- UCHAR       *EFuse    )
This function reads allow user to send 128bytes Efuse table to driver.At the time, driver only save the 128 bytes content in shadow memory. This can increase efuse lifetime.

- int RTLMP_API  WriteEFuseByte(LPADAPTER AdapterObject,

UCHAR offset,

UCHAR Value)
This function writes 1-byte value into the byte offset of EFUSE shadow memory. It will return TRUE/FALSE to show operation success or fail. This can increase efuse lifetime.

- int RTLMP_API	UpdateEFuse (    	LPADAPTER   AdapterObject    )
This function allow user to update previous content in shadow memory to real EFUSE area. Driver will automatically compare shadow table and current efuse content. And we only write different area into efuse to prevent waste of efuse operation.

# Packet TX/RX Functions

- int ResetTxPacketSent(LPADAPTER AdapterObject)
This function resets the packet amount transmitted. It will return TRUE/FALSE to show operation success or fail.

- int QueryTxPacketSent(LPADAPTER AdapterObject, int* TxPacketCount)
This function queries for the packet amount transmitted. It will return TRUE/FALSE to show operation success or fail.

- int ResetRxPacketReceived(LPADAPTER AdapterObject)
This function resets the number of packet received successfully and the number of packet received with CRC32 error. It will return TRUE/FALSE to show operation success or fail.

- int QueryRxPacketReceived(LPADAPTER AdapterObject, int* RxPacketCount)
This function queries for the number of packet received successfully. It will return TRUE/FALSE to show operation success or fail.

- Int QueryRxPacketCRC32Error(	LPADAPTER AdapterObject, int* RxPacketCRC32Error)
This function queries for the number of packet received with CRC32 error. It will return TRUE/FALSE to show operation success of fail.

- PVOID PacketAllocatePacket(LPADAPTER AdapterObject)
This function allocates a buffer for struct PACKET and returns the pointer to PACKET.

- int PacketInitPacket(LPPACKET lpPacket, PVOID Buffer, int Length)
This function initializes a packet object to point to a memory buffer described by Start address and length. It will return TRUE/FALSE to show operation success or fail.

- int PacketSendPacket(LPADAPTER AdapterObject, LPPACKET lpPacket, BOOLEAN Sync)
This function sends out a packet in synchronous mode(Sync:1) or not(Sync:0). It will return TRUE/FALSE to show operation success or fail.

- <Note>

1. lpPacket must be allocated via PacketAllocatePacket() and associated to the buffer of packet to send via PacketInitPacket() before using PacketSendPacket().

- BOOLEAN PacketWaitPacket(LPADAPTER AdapterObject, LPPACKET lpPacket, PULONG BytesReceived, BOOLEAN bWait)
This function queries if one packet has been sent out or not. If you set bWait to be TRUE, then this function will not return until the packet has been sent out. Otherwise, it will return TRUE/FALSE immediately to indicate if the packet has been sent out or not.

- int PacketFreePacket(LPPACKET lpPacket)
This function free a PACKET which is allocated by PacketAllocatePacket().

- <Note>

- We MUST release a PACKET object allocated by PacketInitPacket(), otherwise, memory leak will happen.

- <Example>

// LPADAPTER pAdapter;

// UCHAR PacketContent[128];

LPPACKET PacketS=PacketAllocatePacket(pAdapter);

PacketInitPacket( (PACKET*)PacketS, PacketContent, 128);

PacketSendPacket((_ADAPTER*)Adapter.hFile, (PACKET*)PacketS, TRUE );

PacketFreePacket((PACKET*)PacketS);

# Customer Requirement